Periodica Polytechnica

Chemical Enginering

Chemical Engineering

x(y)pp. x-x, (year)

doi: 10.3311/pp.tr.201x­x.xx

Creative Commons Attribution O

RESEARCH ARTICLE

# Effects of pH and aeration conditions on xylitol production by Candida and Hansenula yeasts

Zoltán Mareczky1,2** / Csaba Fehér1 /Zsolt Barta1 /Katalin Réczey1

Received 201x-xx-xx

Abstract

During the fermentative production of xylitol the following environmental parameters have a controlling effect on the xylitol yield: concentration of monosaccharides, temperature, aeration and pH. The purpose of the present work was to evaluate xylitol production by four yeast strains at different pH values under different aeration conditions. Suitable pH values and aeration conditions have been determined for four yeasts by shake flask fermentations: 
Candida parapsilosis: pH 5.0, OTR 6.10 mmol L-1 h-1 
Candida guilliermondii: pH 4.5, OTR 5.72 mmol L-1 h-1; 
Candida boidinii: pH 6.0, OTR 5.72 mmol L-1 h-1; 
Hansenula anomala: pH 4.5, OTR 2.80 mmol L-1 h-1 using 50 g L-1 initial xylose concentration. The aeration has an impact on both the cell growth and the xylitol yield.

Keywords

xylitol fermentation · Candida and Hansenula yeasts · aeration · shake flasks fermentation

Acknowledgement

The authors gratefully acknowledge the Project OTKA PD-108389 of the Hungarian Research Fund for the financial support. This work is connected to the scientific program of the "Development of quality-oriented and harmonized R+D+I strategy and functional model at BME" project. This project is supported by the New Hungary Development Plan (Project ID: TÁMOP-4.2.1/B-09/1/KMR-2010-0002).

1Department of Applied Biotechnology and Food Science,

Faculty of Chemical Technology and Biotechnology,
Budapest University of Techology and Economics H-1111 Budapest,Hungary

(e­mail: csaba_feher@mail.bme.hu, zsolt_barta@mkt.bme.hu, kati_reczey@mkt.bme.hu)

2GlaxoSmithKline Biologicals Kft., H-2100Gödöllő, Hungary

* Corresponding author, e-mail: mareczky@mail.bme.hu

# Background

Xylitol, a natural five-carbon sugaralcohol, has attracted attention because of its wide variety in use and its good biological impacts [1, 2]. Its sweetness is equal to sucrose, hence xylitol is used increasingly as a sweetener and it has anticariogenic attribute 3. Xylitol raises the blood sugar level less than sucrose, and it can be used as an insulin-independent carbohydrate source for diabetics [4, 5, 6]. Xylitol is also used in chewing gums and toothpastes because it appears to have caries-preventive effect on root surfaces 6[7, 8, 9].

Xylitol is naturally present in some fruits and vegetables (e.g. cauliflower or cabbage lettuce) [10], however its low concentration in plants makes production by extraction difficult and uneconomical 1. On an industrial scale, xylitol has been manufactured by catalytic hydrogenation of D-xylose using e.g. Raney nickel catalysts and this requires expensive purification, in which D-xylose is separated from the other sugars obtained in the acidic hydrolysis of plant hemicelluloses [11, 12].

The bioconversion of xylose by microorganisms is an alternative xylitol production route, a technology with great economic potential. In yeasts D-xylose is converted into D-xylulose by two enzymatic steps [13]. The first phase involves the xylose reductase (XR, E.C.1.1.1.21), which reduces D-xylose into D-xylitol and it depends on either NADH or NADPH cofactors [14]. The subsequent conversion of D-xylitol into D-xylulose is catalyzed by a NAD+-dependent xylitol dehydrogenase (XDH, E.C.1.1.1.9). After a phosphorylation step, D-xylulose can enter the pentose phosphate pathway [14, 15].

The dissolved oxygen concentration is one of the most important environmental parameter in the production of xylitol by yeasts [1, 14, 15]. The oxygen concentration directly affects the yield and the volumetric productivity of xylitol because the generation rate of the necessary cofactors depends on it [16, 17]. It has been reported that the xylose bioconversion to xylitol is strongly affected by the oxygen supply: low dissolved oxygen concentration is necessary for sufficient xylitol production [15, 18, 19]. When it occurs, the terminal oxidation step is unable to generate enough NAD+ from the NADH produced. As a consequence, an increase in intracellular NADH levels occurs, suppressing the reaction catalysed by XDH and allowing xylitol to accumulate [1, 12, 20].

The concentration of different monosaccharides and vitamins also play a major role on the production of xylitol [1, 11,20]. An ideal strategy employed in xylitol biotechnological production is that glucose and arabinose can be used to supply energy and to grow cell concentration, while xylose can be used for xylitol production. However, xylose and arabinose are not metabolized by the most microorganisms until glucose is present in the medium [11]. This regulation called carbon catabolite repression (CCR), prevents the uptake and catabolism of less favorable carbon sources [11, 15, 20, 21]. The pH value is also important for the optimal production of xylitol. The yeasts prefer the pH values between 4 and 6 [15, 22, 23].

The purpose of the present work was to evaluate the xylitol and cell production during the xylitol fermentation by strains Candida guilliermondii (Pichia guilliermondii), Candida boidinii, Candida parapsilosis and Hansenula anomala (Pichia anomala) at various pH values and different aeration conditions. These different aeration conditions were obtained by varying the liquid volumes (filling of the flasks) in the 100 mL Erlenmeyer flasks.

# Materials and methods

# Strains and inoculum culture conditions

The experiments were carried out with the following strains: C. boidinii (NCAIM Y.01308); C. guilliermondii (NCAIM Y.01050); C. parapsilosis (NCAIM Y.01011); and H. anomala (NCAIM Y.01499), obtained from National Collection of Agricultural and Industrial Microorganisms (Budapest, Hungary). The cultures were maintained on glucose agar slant at 4-8°C. The cells, aged for 6-10 days, were transferred to 750 mL Erlenmeyer flasks containing 100 mL of the medium: 30 g L-1 D-glucose; 10 g L-1 yeast extract; 15 g L-1 KH2PO4; 3 g L-1  (NH4)2HPO4; and 1 g L-1 MgSO4 x 7H2O. The flasks were incubated in a rotary shaker (220 rpm) at 30 °C for 24 hours.

# Fermentation conditions

In those cases, when the aim of the experiment was the examination of the effect of pH, the fermentations were carried out in 100 mL Erlenmeyer flasks with 50 mL medium containing: 50 g L-1 D-xylose; 10 g L-1 yeast extract; 15 g L-1 KH2PO4; 3 g L-1 (NH4)2HPO4 and 1 g L-1 MgSO4 x 7H2O. The pH value of the fermentation media was adjusted to 4.5; 5.0; 5.5; 6.0 at each strain. The pH was monitored and controlled during the fermentations with sterile 10% NaOH and 10% HCl. The agitation speed was set to 125 rpm.

After these experiments the next (aeration-condition-oriented) fermentations were performed on a rotary shaker at 220 rpm on that pH value which resulted in the highest xylitol yield before. These experiments were carried out in 100 mL Erlenmeyer flasks with 30/40/50/60 mL medium (Filling ratio 0.3/0.4/0.5/0.6). To determine the aeration capacities of the shaked flasks, the volumetric mass-transfer coefficient of oxygen (KLa) was measured with non-fermentative gassing-out method. The dissolved oxygen concentration of the solution (C) is lowered by gassing the liquid out with nitrogen gas [24]. The deoxygenated liquid is then agitated and the increase of dissolved oxygen concentration due to the shaking of the flask was measured until a constant level (C*). The measurements were performed at 30°C by using different levels of medium volume (30, 40, 50 and 60 mL) and rotation speed (125 and 220 rpm). The values of the expression –ln[1-(C/C*)] was plotted as a function of time. The slope of the fitted linear curve gave the value of KLa (h-1). Maximum possible oxygen transfer rate (OTR) was calculated by multiplying KLa (h-1) and C*(mmol L-1). The oxygen concentration was monitored using Hamilton Bonaduz AG (Switzerland) VisiFerm® DO (ECS) 120 online dissolved oxygen sensor.

The initial cell mass was 4.0 g L-1 during all shake flask fermentations. Flasks were closed with cotton wool plugs and paper caps. All experiments were performed with daily sampling at 30 °C for 96 hours.

# Analytical methods

The fermentations were monitored through periodic sampling to determine xylose uptake, xylitol production and cell growth. Samples of an appropriate dilution were prepared by filtration through 0.45 µm regenerated cellulose membrane before injection for chromatography analysis. Xylose and xylitol were analysed by high-performance liquid chromatography (Shimadzu LC-VP) using refractive index detector and  a BIO-RAD Aminex HPX-87H column at 65 °C and degasified 5 mM sulfuric acid (diluted with ultrapure water) solution as mobile phase at a flow rate of 0.5 mL min-1.The xylitol yield was calculated from the highest obtained xylitol concentration [g L-1] and the theoretical maximum based on the initial xylose concentration [g L-1] see Eq. (1).

Xylitol yield = highest xylitol concentration [gL]initial xylose concentration [gL]×152 [gmol]150 [gmol]×100 [%](Eq.1)

Cell growth was estimated by optical density measurement at 600 nm [25]. The cell concentration was determined using standard curves of absorbance vs. cell mass concentration.

# Results and discussion

# pH experiments

In the first series of experiments the objective was to identify the appropriate pH value for the four yeast strains. Figure 1 shows the xylitol yield obtained during the pH experiments. During the pH experiments the OTR was 2.8 mmol L-1h-1.

Xylitol yield (% of theoretical) during the pH experiments [filling: 50 mL / 100 mL; Rotary speed: 125 rpm; OTR 2.8 mmol L-1 h-1]. The standard deviations were calculated from triplicates.

H. anomala showed the best performance under these conditions among the four strains. Its xylitol yields were higher than those of the other strains at all pH values, and its maximum yield (47.8%) was obtained at pH 4.5. Tao et al. [26] evaluated the optimal pH range for H. anomala and it was observed that pH 4.5 was the most eligible pH value for this strain.

The xylitol yield was 24.2% in the case of C. guilliermondii at pH 4.5 and this yield decreased at the other pH values. Felipe et al. [27] also reports that pH 4.5 is appropriate for xylitol production by C. guilliermondii. However other studies demonstrated that the most eligible pH for this strain is 5.0 [28] and 5.5 [15].

As shown in Figure 1, the xylitol yield by C. boidinii increased by increasing the pH, the highest final xylitol yield (31.5%) was observed at pH 6.0. Sakai et al. [29] also documented that pH 6.0 is eligible for C. boidinii. In the case of C. parapsilosis the maximum xylitol yield (35.8%) was obtained at pH 5.0, however there is no significant difference between the xylitol yields at pH 5.0 and 5.5. The yield decreased at pH 4.5 and 6.0 in the case of this strain (Figure 1). Kim et al. [3030] also reported that pH 5.0-5.5 is appropriate for C. parapsilosis. However Nolleau et al. [31] demonstrated that the most eligible pH for this strain is 4.75.

Results in Table 1 indicated that in the case of H. anomala the highest xylitol concentrations (21.7 g L-1; 19.0 g L-1 and 19.3 g L-1) were obtained at 96 hours at pH 4.5; pH 5.0 and pH 5.5, respectively. In these cases the calculated productivity was 0.23, 0.20 and 0.21 g L-1h-1, respectively. However, at 72 hours higher productivities, 0.23 and 0.24 g L-1h-1 were observed in the case of C. boidinii at pH 5.5 and 6.0, respectively (Table 1). This strain obtained its maximum xylitol concentration (17.5 g L-1) 24 hours earlier than H. anomala, hence the productivity of C. boidinii is the highest (Table 1). The results show that the productivity decreased from 0.12 g L-1h-1 to 0.07 g L-1h-1 in the case of C. guilliermondii as the pH value increased.

TABLE 1 Maximum xylitol concentrations, corresponding cultivation times and volumetric productivities during the pH experiments (initial xylose concentration was 50 g L-1)

|  |  | Candida boidinii |  |  | Candida guilliermondii |  |  | Candida parapsilosis |  |  | Hansenula anomala |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| pH | Xylitol conc. [g L-1] |  | Time [h] | Productivity [g L-1h-1] | Xylitol conc. [g L-1] | Time [h] | Productivity [g L-1h-1] | Xylitol conc. [g L-1] | Time [h] | Productivity [g L-1h-1] | Xylitol conc. [g L-1] | Time [h] | Productivity [g L-1h-1] |
| 4.5 |  | 12.2 | 96 | 0.13 | 11.2 | 96 | 0.12 | 13.7 | 96 | 0.14 | 21.7 | 96 | 0.23 |
| 5.0 |  | 13.8 | 96 | 0.14 | 7.8 | 96 | 0.08 | 16.7 | 96 | 0.17 | 19.0 | 96 | 0.20 |
| 5.5 |  | 16.4 | 72 | 0.23 | 7.5 | 96 | 0.08 | 15.5 | 96 | 0.16 | 19.3 | 96 | 0.21 |
| 6.0 |  | 17.5 | 72 | 0.24 | 6.7 | 96 | 0.07 | 11.1 | 96 | 0.12 | 17.4 | 96 | 0.18 |

OTR: 5.72

OTR: 6.10

The cell mass concentrations are presented in Table 2. We observed that the yeasts reached the highest cell mass concentrations at those pH values where the highest final xylitol concentrations were obtained. There is a relation between the maximum xylitol yield and final cell mass concentration. The higher the cell mass concentration the higher the xylitol concentration is (Table 1 and Table 2).

OTR: 6.25

OTR: 6.48

The final cell mass concentration results show that the C. boidinii achieved lower cell mass as the pH decreased, in this case the cell concentration was 5.0 g L-1 at pH 4.5.

We observed that C. guilliermondii achieved the highest final cell mass (8.6 g L-1 at pH 5.0) from the 4.0 g L-1 initial cell count.

TABLE 2 Final cell mass concentrations during the pH experiments

| pH |  | Final cell mass concentrations [g L-1] |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | C. boidinii |  | C. guilliermondii |  | C. parapsilosis | H. anomala |
| 4.5 |  | 5.0 | 8.5 | 8.0 |  | 8.2 |
| 5.0 |  | 5.8 | 8.6 | 8.0 |  | 7.4 |
| 5.5 |  | 6.4 | 7.8 | 7.3 |  | 7.5 |
| 6.0 |  | 7.0 | 7.6 | 7.1 |  | 7.2 |

# Aeration-condition-oriented experiments

Figure 2 shows the xylitol yields obtained during the aeration-oriented fermentations. These experiments were performed on that pH value which resulted in the highest xylitol yield before.

At filling ratio 0.3 and 0.4 the OTR was 6.48 and 6.25 mmol L-1h-1 respectively. The four investigated strains achieved low xylitol yields under these aeration conditions (Figure 2). In the case of C. boidinii, C. guilliermondii and C. parapsilosis the xylitol yields varied between 43.2 and 46.9% at filling ratio 0.5 and 0.6 with oxygen transfer rate 6.10 and 5.72 mmol L-1h-1 respectively.

Xylitol yield (% of theoretical) during the during aeration-condition-oriented fermentations [Rotary speed: 220 rpm]. The standard deviations were calculated from triplicates.

A significant difference was observed between the theoretical xylitol yields obtained at filling ratio 0.3-0.4 and 0.5-0.6. The most probable reason of this result is the difference between the oxygen transfer rates. The assessed yeasts obtained lower xylitol yield as a result of higher dissolved oxygen concentration when the OTR was 6.48 or 6.25 mmol L-1h-1. However the lower oxygen concentrations resulted higher xylitol yields in all cases, when the OTR was 6.10 or 5.72 mmol L-1h-1.

Higher xylitol yields were obtained by C. boidinii, C. guilliermondii and C. parapsilosis with 50 mL (OTR was 6.10 mmol L-1h-1) fermentation bulk volume at 220 rpm than at 125 rpm during the pH experiments when the OTR was 2.8 mmol L-1h-1 (Figures 1 and 2).

However, H. anomala obtained 13.2% xylitol yield at filling ratio 0.5 with 220 rpm and OTR 6.10 mmol L-1h-1. This yield is significantly lower than during the pH experiments, when this strain obtained 46.9% xylitol yield at filling ratio 0.5 and with 125 rpm rotary speed and OTR 2.8 mmol L-1h-1.

TABLE 3 Maximum xylitol concentrations during the aeration-oriented fermentations

| Filling [mL / 100 mL] |  | Candida boidinii |  |  | Candida guilliermondii |  |  | Candida parapsilosis |  |  | Hansenula anomala |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Xylitol conc. [g L-1] |  | Time [h] | Productivity [g L-1h-1] | Xylitol conc. [g L-1] | Time [h] | Productivity [g L-1h-1] | Xylitol conc. [g L-1] | Time [h] | Productivity [g L-1h-1] | Xylitol conc. [g L-1] | Time [h] | Productivity [g L-1h-1] |
| 30 |  | 21.5 | 48 | 0.45 | 6.3 | 96 | 0.07 | 5.5 | 72 | 0.08 | 6.4 | 72 | 0.09 |
| 40 |  | 20.3 | 48 | 0.42 | 6.3 | 96 | 0.07 | 6.9 | 96 | 0.07 | 5.3 | 96 | 0.06 |
| 50 |  | 23.6 | 48 | 0.49 | 23.0 | 96 | 0.24 | 25.2 | 96 | 0.26 | 6.0 | 96 | 0.06 |
| 60 |  | 23.7 | 48 | 0.49 | 23.2 | 96 | 0.24 | 23.1 | 96 | 0.24 | 8.7 | 96 | 0.09 |

The eligible aeration condition for the H. anomala yeast was obtained with OTR 2.8 mmol L-1h-1, however the strains C. parapsilosis, C. boidinii and C. guilliermondii achieved the highest xylitol yields with oxygen transfer rates 5.72 and 6.10 mmol L-1h-1.

In the case of C. boidinii maximum xylitol concentrations were obtained at all volumes at 48 hours and the productivity was 0.49 g L-1h-1 at filling ratio 0.5 and 0.6 (Table 3). Although similar maximum xylitol concentrations were obtained by the other strains at filling ratio 0.5 and 0.6, except H. anomala, the productivities were lower, since longer fermentation times were required (Table 3).

TABLE 4 Final cell mass concentrations during the aeration-oriented fermentations

| Filling [mL/100mL] |  | Final cell mass concentrations [g L-1] |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | C. boidinii |  | C. guilliermondii |  | C. parapsilosis | H. anomala |
| 30 |  | 7.8 | 11.0 | 10.6 |  | 14.0 |
| 40 |  | 8.8 | 10.3 | 8.6 |  | 11.5 |
| 50 |  | 7.9 | 14.0 | 12.0 |  | 10.2 |
| 60 |  | 7.8 | 11.4 | 8.9 |  | 10.5 |

High final cell mass concentrations were obtained by H. anomala, however it produced xylitol at low concentrations (Table 3). The final cell mass concentration increased by decreasing the filling volume, which is in line with the fact that lower filling volumes results in better oxygen supply (OTR30mL= 6.48 mmol b L-1h-1 and OTR40mL= 6.25 mmol L-1h-1).

Under the discussed conditions in the case of C. boidinii, C. guilliermondii and C. parapsilosis there is not a tendency in the obtained final cell mass concentrations under different aeration conditions.

# Conclusion

During our work the suitable pH values and aeration conditions have been determined for four yeast strains: H.anomala: pH 4.5, 125 rpm with filling ratio 0.5 (OTR 2.80 mmol L-1 h-1); C.parapsilosis: pH 5.0, 220 rpm with filling ratio 0.5 (OTR 6.10 mmol L-1 h-1);   C. guilliermondii: pH 4.5, 220 rpm with filling ratio 0.6 (OTR 5.72 mmol L-1 h-1) and C. boidinii: pH 6.0, 220 rpm with filling ratio 0.6 (OTR 5.72 mmol L-1 h-1) using 50 g/L initial xylose concentration. On the contrary Vandeska et al. [32] reported that the eligible oxygen transfer rate is 14 mmol L-1h-1 for C. boidinii strain using 130 g L-1 initial xylose concentration. On the contrary Feher et al. [33] found that the appropriate aeration conditions for C. boidinii was obtained with 2.8 mmol L-1h-1 using 30 g L-1 initial xylose during the fermentations. Based on this observation it can be presumed that using this yeast the higher the initial xylose concentration the higher the OTR is required.

During our work the obtained xylitol yields were between 45 and 48% of the theoretical maximum, which is comparable to the yields reported elsewhere [16].

Compared to the literature it can be concluded that the appropriate pH and aeration for the yeasts also depends on that, which isotype of strains are used, hence it is of importance to determine the most favourable conditions for each isotypes of a certain strain.

References

Silva S.S., Chandel A. K."D-Xylitol – Fermentative production, application and commercialization” Springer-Verlag, Berlin.

- (2012).

- DOI: 10.1007/978-3-642-31887-0

Pérez-Bibbins B.,Torrado-Agrasar A., Salgado J. M., Mussatto S. I., Domínguez J. M. “Xylitol production in immobilized cultures: a recent review” Critical Reviews in Biotechnology 35 (x), pp. 1-14. (2015).

- DOI: 10.3109/07388551.2015.1004660

Gazzani G., Daglia M., Papetti A.“Food components with anticariesactivity.” Current Opinion in Biotechnology 23 (2), pp. 153–159. (2012).

- DOI: 10.1016/j.copbio.2011.09.003.

Amo K., Arai H., Uebanso T., Fukaya M., Koganei M., Sasaki H., Yamamoto H., Taketani Y., Takeda E.“Effects of xylitol on metabolic parameters and visceral fat accumulation.” Journal of Clinical Biochemistry and Nutrition 49, pp. 1-7. (2011).

- DOI:10.3164/jcbn.10-111

Azarpazhooh A., Limeback H., Lawrence H. P., Shah P. S.“Xylitol preventing acute otitis media in children up to 12 years of age.” Cochrane Database of Systematic Reviews (11:CD007095).

- (2011).

- DOI:10.1002/14651858.CD007095

Islam M. S. “Effects of xylitol as a sugar substitute on diabetes-related parameters in nondiabetic rats.” Journal of Medicinal Food 14, pp. 505–511.

- (2011).

- DOI:10.1089/jmf.2010.0015

Bader J. D.,Shugars D. A., Vollmer W. M., Gullion C. M., Gilbert G. H., Amaechi B. T., Brown J. P. “Design of the xylitol for adult caries trial (X-ACT).” BMC Oral Health 10, pp. 22–29. (2010).

- DOI:10.1186/1472-6831-10-22

Ritter A.V., Bader J. D., Leo M. C., Preisser J. S., Shugars D. A., Vollmer W. M., Amaechi B. T., Holland J. C. “Tooth-surface-specific effects of Xylitol: Randomized Trial Results.”Journal of Dental Research 92, pp.512-517.

- (2013).

- DOI: 10.1177/0022034513487211

Hildebrandt G., Lee I., Hodges J. “Oral mutants streptococci levels following use of a xylitol mouth rinse: a double-blind, randomized, controlled clinical trial.”Special care in Dentistry 30, pp. 53-58.

- (2010).

- DOI: 10.1111/j.1754-4505.2009.00124.x.

Chen X., Jiang Z-H., Chen S., Qin W.“Microbial and Bioconversion Production of D-xylitol and its detection and application.”International Journal of Biological Sciences 6 (7), pp.834-844.

- (2010).

- DOI: 10.7150/ijbs.6.834

Su B., Wu M., Lin J., Yang L. “Metabolic engineering strategies for improving xylitol production from hemicellulosic sugars.” Biotechnology Letters 35(11), pp. 1781-1789.

- (2013).

- DOI: 10.1007/s10529-013-1279-2

Granström T. B., Izumori K., Leisola M.“A rare sugar xylitol. Part I-II: the biochemistry and biosynthesis of xylitol – Biotechnological production and future applications of xylitol.”Applied Microbiology and Biotechnology 74, pp. 277-281.

- (2007).

- DOI: 10.1007/s00253-006-0760-4.

Hirabayashi S., Wang J., Kawagishi H., Hirai H.“Improving xylitol production through recombinant expression of xylose reductase in the white-rot fungus Phanerochaetesordida YK-624.”Journal of Bioscience and Bioengineering” 118(x), pp. 1-3. (2014).

- DOI: 10.1016/j.jbiosc.2014.11.024.

Aranda-Barradas J. S., Garibay-Orijel C., Badillo-Corona J. A., Salgado-Manjarrez E. ”Astochiometric analysis of biological xylitol production.” Biochemical Engineering Journal 50 (1-2), pp. 1-9.

- (2010).

- DOI: 10.1016/j.bej.2009.10.023

Albuquerque T. L., Silva I. J. Jr., Macedo G. R., Rocha M. V. P. “Biotechnological production of xylitol from lignocellulosic wastes: A review.” Process Biochemistry 49, pp. 1779-1789.

- (2014).

- DOI: 10.1016/j.procbio.2014.07.010

Rodrigues R. C. L. B., Sene L., Matos G. S., Roberto I. C., Pessoa Jr. A., Felipe M. G. A.“Enhanced Xylitol Production by Precultivation of Candida gulliermondii Cells in Sugarcane Bagasse Hemicellulosic Hydrolysate.”Current Microbiology 53., pp. 53-59.

- (2006).

- DOI: 10.1007/s00284-005-0242-4

Felipe M. G. A., Vitolo M., Mancilha I. M., Silva S. S. “Environmental parameters affecting xylitol production from sugar cane bagasse hemicellulosichydrolyzate by Candida guilliermondii.”Journal of Industrial Microbiology & Biotechnology 18, pp. 251-254.

- (1997).

- DOI: 10.1038/sj.jim.2900374

Faria L. F. F., Gimenes M. A. P., Nobrega R., Pereira N. Jr. “Influence of oxygen availability on cell growth and xylitol production by Candida guilliermondii.”Applied Microbiology and Biotechnology 98-100, pp.449-458.

- (2002).

- DOI: 10.1385/ABAB:98-100:1-9:449

Yablochkova E. N., Bolotnikova O. I., Mikhailova N. P., Nemova N. N., Ginak A. I. “The activity of Xylose Reductase and Xylitol Dehydrogenase in Yeasts.”Microbiology. 72, pp.414-417.

- (2003).

- DOI: 10.1023/A:1025032404238

Cheng H., Wang B., Lv J., Jiang M., Lin S., Deng Z.”Xylitol production from xylose mother liquor: a novel strategy that combines the use of recombinant Bacillus subtilis and Candida maltose”. Microbial Cell Factories 10 (5),pp. 1-12.

- (2011).

- DOI: 10.1186/1475-2859-10-5

Misra S., Gupta P., RaghuwanshiS., Dutt K., Saxena R. K.”Comparative study on different strategies involved for xylitol purification from culture media fermented by Candida tropicalis”. Separation Purification Technology 78 (3), pp. 1-8.

- (2011).

- DOI: 10.1016/j.seppur.2011.02.018

Ghindea R., Csutak O., Stoica I., Tanase A-M., Vassu T. “Production of xylitol by yeasts.”Romanian Biotechnological Letters 15 (3), pp. 5217-5222.

- (2010).

- DOI:10.1186/1475-2859-6-5

Srivani K., PydiSetty Y.“Parametric optimization of xylitol production from xylose by fermentation.”Asia-Pacific Journal of Chemical Engineering 7 (3), pp.280-284.

- (2012).

- DOI: 10.1002/apj.1645

Taguchi H., Humphrey, A. E. “Dynamic Measurement of the Volumetric Oxygen Transfer Coefficient in Fermentation Systems” Journal of Fermentation Technology 44, pp. 881-889

- (1966).

Zhang J., Geng A., Yao C., Lu Y., Li Q. “Xylitol production from D-xylose and horticultural waste hemicellulosic hydrolysate by a new isolate of Candida athensensis SB18” Bioresource Technology 105, pp. 134-141.

- (2012).

- DOI: 10.1016/j.biortech.2011.11.119

Tao N., Gao Y., Liu Y. “Isolation and characterization of a Pichia anomala strain: a promising candidate for bioethanol production” Brazilian Journal of Microbiology 42 (2), pp. 668-675.

- (2011).

- DOI: 10.1590/S1517-838220110002000031

Felipe M. G. A., Vitolo M., Mancilha I. M.,  Silva S. S. “Fermentation of sugar cane bagasse hemicellulosic hydrolysate for xylitol production: effect of pH” Biomass and Bioenergy 13 (1), pp. 11-14.

- (1997).

- DOI: 10.1016/S0961-9534(97)00032-9

Rubio C., Latina C., Navarro A. “Fermentation of corncob hydrolysate for xylitol production.” BioTechnología 16 (3), pp. 48-63.

- (2012).

- DOI: 20121603_48-63

Sakai Y., Marshall P. A., Saiganji A., Takabe K., Saiki H., Kato N., Goodman J. M. “The Candida boidinii Peroxisomal Membrane Protein Pmp30 has a Role in Peroxisomal Proliferation and Is Functionally Homologous to Pmp27 from Saccharomyces cerevisiae.” Journal of Bacteriology 177 (23), pp. 6773-6781.

- (1995).

- DOI: 0021-9193/95

Kim S. Y., Oh D. K., Choi J. H., Kim S. E. “Fermentation process for preparing xylitol using mutant cells.” United States Patent No. 5.686.277

- (1997).

- DOI: US5686277 A

Nolleau V., Preziosi-Belloy L., Navarro J. M. “The reduction of xylose to xylitol by Candida guilliermondii and Candida parapsilosis: incidence of oxygen and pH.” Biotechnology Letters 17 (4), pp. 417-422.

- (1995).

- DOI: 10.1007/BF00130800

Vandeska E., Kuzmanova S., Jeffries T. W. “Xylitol formation and key enzyme activities in Candida boidinii under different oxygen transfer rates” Journal of Fermentation and Bioengineering 80 (5), pp. 513-516.

- (1995).

- DOI: 10.1016/0922-338X(96)80929-9

Feher Cs., Gazsó Z., Gál B., Kontra A., Barta Zs., Réczey K. “Integrated process of arabinose biopurification and xylitol fermentation based on the diverse action of Candida boidinii” Chemical and Biochemical Engineering Quarterly (under publishing)

- (2015).